# Register

## IO

| Name | Size | Input/Output | Function |
| --- | --- | --- | --- |
| clk | 1 | input | clock |
| rst | 1 | input | Reset register |
| d | 1 | input | Data into the register |
| q | 1 | output | Output data |
| set | 1 | input | Pre-sets the data if we want to store data before |

## Reset

Reset is synchronous and signaled by setting rst high for at least 1 clock cycle. When the reset is complete, the register will be set LOW.

## set

Reset is synchronous and signaled by setting set high for at least 1 clock cycle. When the set is complete, the register will be set HIGH or LOW based on the set data.

# ROM

## IO

| Name | Size | Input/Output | Function |
| --- | --- | --- | --- |
| clk | 1 | input | clock |
| addr | 1 | input | Selecting the register to take data bits from |
| q | 1 | output | Output data |

## Address

We will read memory through a mux reading the address from the outputs of the shift registers.

System Verilog

module rom (

input logic clk,

input logic[15:0] addr,

output logic[47:0] q);

// Declare the ROM as an array

logic [47:0] rom[65536];

initial // Read the ROM contents from a file

begin

$readmemb("rom_init.txt", rom);

end

// Access the ROM

assign q = rom[addr];

endmodule