# Team25 Saturation Interface

Work by Gutten på Hemsen; CC BY-SA 4.0

Output from convolution with a filter can produce signals that clip, with a number of bits larger than available further along the data path. As such, a saturation module is needed to ensure digital audio signals are saturated at too high or low signals - preventing seemingly unpredictable output.

## IO

| Name | Size | Input/Output | Function |
| --- | --- | --- | --- |
| ena | 1 | input | Enable module. |
| d_in | N | input | Signed digital audio data in. |
| d_out | M | output | Signed digital audio data out. M ≤ N. |
| clip | 1 | output | High if clipping high, low if clipping low |
| slice | 2^(N-M+1) | input | Lowest bit of the slice from d_in. |

## Typical Configuration

b

## Input/Output Size

Input and output bit sizes are arbitrary and parameterized. However, the size of d_out must be smaller than the size of d_in.

## Slice

## Theory of Operation

The saturate module checks for overflow and underflow of the N bits of d_out. If d_in is positive, check for 1s in the upper N-M bits of d_in. Then, if there are any 1s, output d_out = maximum (2^M-1). If d_in is negative, check for 0s in the upper N-M bits of d_in. Then, if there are any 0s, output d_out = minimum (M’b0).